ISyE 6201: Manufacturing Systems

Instructor: Spyros Reveliotis

Spring 2007

Homework #1

Due Date: Tuesday, 2/13/07

Reading Assignment:

- Sections 2.1 to 2.3 and 17.1 to 17.3 from your textbook.

- The last three items in the list posted at the Library Electronic Reserves.

Problem set:

- Solve Problems 4-7 and 9 in Chpt. 2 of your textbook, and also Problem 1 in Chpt. 17.

- Consider the variation of the basic EOQ model which allows a maximum backorder at every cycle of B units, i.e., at the beginning of every cycle the inventory position (IP) is at –B units, an order for Q units is placed and delivered instantaneously, which bring the IP up to the Q-B level, and subsequently, this quantity is depleted with a rate of D units/time unit, until a backorder of B units is built again, at which point, a new cycle begins. Furthermore, in addition to the ordering, item and holding costs, A, C, and h, now we must also define a backordering cost b, measured in $ per unit backordered per time unit. Your tasks are the following:

  - Determine the optimal pair (Q*,B*) that minimizes the total operational cost per time unit. (Hint: You have to work as we did in class, by expressing the various components of the total cost per time unit as a function of Q and B, and then minimize the resulting expression by taking the derivatives with respect to these two variables and setting them equal to 0.)

  - Consider whether it is possible to derive from your results for part a, the basic EOQ formula derived in class.