ISyE 6201: Manufacturing Systems

Instructor : Spyros Reveliotis

Spring 2007

Solutions for Homework #2

A.

Chapter 2

12. The chairs are made in-house and so we are attempting to determine the appropriate parameters for a base-stock system. We assume that the wholesalers order once per month.

(a) The holding cost is h=$5 while the backorder cost is b=$20. The distribution of demand during a month is well approximated by a normal distribution with a mean of 1,000 chairs and a standard deviation of 200 chairs. Then, if X represents the demand during one month,

G(R*) = b/(h+b) = 20/(5+20) = 0.8

and so

G(R*) = P{X≤R*} = P{(X-1000)/200 ≤ (R*-1000)/200} = 0.8

The value of the standard normal with 0.8 probability is obtained from a standard normal table or using the Excel function NORMSINV(0.8) and yields 0.84. Then the order up to point is computed as

R* = 1000+0.84(200) = 1168.

(b) If the sale is lost (as opposed to backordered ) then the shortage cost must be the profit that would have been made which is $100. The computation is then similar,

G(R*) = 100/(5+100) = 0.9524

z0.9524 = 1.67

R* = 1000+1.67(200) = 1334

(c) Since the cost of being short is higher in the second case, we want to carry more inventory to avoid that possibility.

14.

(a)

Formulae for some of the quantities:

Dl					Dθbecause demand is POISSON)

F = DQ					I = [R -   + B(R)] c = [r+1 -   + B(r+1)] c

S(R) = G(R-1) = G(r)

Holding cost per year = 12*hI 	Order cost per year = 12*FA

The colored cells are looked up from the table below, which uses the following formulae for the basestock model:

p(R) = Re-R! 			 (cdf of Poisson random variable)

G(R) = k=0Rp(k)			 (by definition on pg. 69 of the textbook)

B(R) = p(R) + {[R] [1-G(R)]} 	 (eqn 2.63 on pg. 100)

(b) EOQ = 2ADh , so QA = 4,  QB = 26.

Using the previous table, we compute the fill rates for the (Q,r) model using

S(Q, r) =1−1Q[B(r)−B(r+Q)] 	 (eqn 2.35 on pg. 78)

To achieve a fill rate of at least 98%, the minimum r for Type A and Type B modules are 12 and 18 units, respectively.

Now since Q>1, the inventory investment and backorder level are computed by

I(Q,r)*c = [(Q+1)/2 + r -  + B(Q,r)]*c

and	B(Q,r) = 1Qx=r+1r+QB(x)

(These two formulae apply to parts (c) and (d) as well since the order quantities Q are greater than 1.)

The higher values of Q make it possible to achieve the same service with lower r values. The inventory is higher due to increased cycle stock caused by bulk ordering. However, the total cost is reduced by about 75% due to the reduction in order cost.

(c) In the backorder model, we find G(R*) using G(R*) = b/(b+h). This critical ratio defines the z-value in the same way as we did in Problem 2-10. Using r* = + z, the reorder points are found to be 9 and 22 for A and B, respectively. Then, we can look up  the Si values from the fill rates table, and we get

This change lowers service for part A (expensive one) and raises it for part B, so the same average service is achieved with lower total inventory. Note that it is even below the base stock inventory level where Q = 1.

(d) Standard deviation of lead time demand lσD2+D2σL2, where L is the standard deviation of lead time. Note that D and L must use the same base time unit (e.g. D = 7 units/month, L= 0.25 month). With the new the reorder points are recomputed using the same formula r* = + zas in part (c).

The variability in the lead times inflates the reorder points – in this case for part B (rounding can result in no change). It should also be pointed out that the predictions of service, backorder level, and inventory level are no longer exact, since the employed formulae were derived under the assumption of fixed lead times.

15. Formulae for some of the quantities:

Dl					θbecause demand is POISSON)

F = DQ					I(Q,r)*c = [(Q+1)/2 + r -  + B(Q,r)]*c

Holding cost per year = 12*hI 	Order cost per year = 12*FA

The fill rates table is at the end of this problem’s solution.

(a,b)

As we observed in class, Type 1 service specifications are most stringent than the corresponding Type 2 ones. Therefore, when such a specification is used as an approximation for fill rate, which is another term for the Type 2 service level, it will underestimate its true value, leading to a much larger r and higher inventory.

(c)

This approximation is very accurate because it is based on the actual formula that characterizes fill rate, and when Q is this large, the dropped term B(r+Q) is negligible.

Note that when Q is reduced, we get slightly higher service at a much smaller inventory investment. But of course, we order twice as often. If we neglect the cost or capacity considerations of placing orders, we can always minimize inventory costs y choosing Q=1. But if we consider either order frequency (capacity) or fixed order cost, then EOQ may give a perfectly reasonable Q.

Formulae used in the fill rates table:

p(r) = re-r! 			(cdf of Poisson random variable)

G(r) = k=0rp(k)			(by def on pg. 69 of the textbook)

B(r) = p(r) + {[r] [1-G(r)]}  (eqn 2.63 on pg. 100. This is the backorder level formula

for the base stock model. The values of B(r) are

computed because they are used in the following B(Q,r)

formula, which is a (Q,r) model formula.)

B(Q, r) =1Qx=r+1r+QB(x) 	 	(eqn 2.38 on pg. 78)

Type 1 service = G (r)		(eqn 2.36 on pg. 78)

Type 2 service = 1−B(r)Q			(eqn 2.37 on pg. 79)

Exact S(Q,r) = 1−1Q[B(r)−B(r+Q)]  	(eqn 2.35 on pg. 78)

Fill rates table for Problem 2.15:

## Multi-product Newsboy Problem:

i. Solution Methodology

In this multi-product newsboy problem, the objective is to find order quantities Qi for newspaper i, i=1,…,N, such that the daily profit is maximized while the total weight of newspapers Qiwi is less than or equal to W.

Maximizing the profit is equivalent to minimizing the total cost, which consists of the underage and overage costs. Let ui = pi - ci be the unit underage cost and oi = ci - si be the unit overage cost for newspaper i. Then the cost contributed by product i is

Ci (Qi, Xi) = oi max{0, Qi - Xi} +  ui max{0, Xi - Qi}

Taking the expected value of the cost with respect to the demand Xi and summing up for the N products, the optimization problem (P) can be formulated as

Minimize

i=1Noi(Qi−x)gi(x)dx+ui(x−Qi)gi(x)dx

subject to

i=1NQiwi≤W

where gi(x) is the probability density function of the demand Xi.

Note that in the basic newsboy model, where there is no weight constraint, Qi=uioi+ui gives separate optimal ordering quantity for newspaper i. So if the combination of these quantities does not violate the weight constraint, i.e. i=1Nuioi+uiwi≤W, then it is a feasible and optimal solution to (P). If i=1Nuioi+uiwi>W, then the optimal solution will satisfy the weight constraint as equality. The reason is that the expected cost of each newspaper, E[Ci (Qi, Xi)] = oi(Qi−x)gi(x)dx+ui(x−Qi)gi(x)dx, is a convex function in Qi. For 0≤Qi<uioi+ui, E[Ci (Qi, Xi)] is decreasing in Qi. For any solution that gives a total weight strictly less than W, the objective function can be improved by increasing some of the Qi’s until the weight constraint is satisfied at equality.

Assuming i=1Nuioi+uiwi>W, we can replace the inequality sign with equality in the constraint in (P) and obtain the same optimal solutions. In this case we may introduce a Lagrange multiplier  and find the optimal solution to (P) by solving the unconstrained problem:

Minimize

C(Q1,...,QN,θ)=i=1Noi(Qi−x)gi(x)dx+ui(x−Qi)gi(x)dx+θi=1NwiQi−W

The optimality conditions are:

∂C∂Qi=oiGi(Qi)−ui+uiGi(Qi)+θwi=0,∀i=1,...,N  	…… (1)

and 		∂C∂θ=i=1NwiQi−W=0					…… (2)

From (1), we have Gi(Qi)=ui−θwioi+ui, so we can write Qi in terms of as

Qi(θ)=Gi−1ui−θwioi+ui.

Then the problem becomes finding a value of such that Qi*() satisfies (2).  can be solved using bisection search over the interval between a lower bound and upper bound for . Note that can be interpreted as the penalty cost of violating the weight constraint by one unit, so a lower bound for is 0. Also, since Gi is a cumulative distribution function, has to satisfy 0≤ui−θwioi+ui≤1, i.e. −oiwi≤θ≤uiwi. Therefore, miniuiwi can be taken as the initial upper bound for in the bisection search.

During bisection search, set  to be the midpoint between the upper and lower boundsStop if 0≤W−i=1NwiQi(θ)<miniwi. Qi(θ) is optimal.  If W−i=1NwiQi≥miniwi, replace the upper bound with the current value of If W−i=1NwiQi<0, replace the lower bound with the current value of Repeat until an optimum is found.

ii. Application (Remark: Notice that in the following calculations, the notation N(a,b) employed in the problem data, has been interpreted as a normal distribution with mean equal to a and st. deviation equal to b.)

| i | ui | oi | critical ratio = ui/(ui+oi) |
| --- | --- | --- | --- |
| 1 | 0.75 | 0.15 | 0.83 |
| 2 | 1.00 | 0.40 | 0.71 |
| 3 | 1.30 | 0.60 | 0.68 |

| Iteration | Lower | Upper |  | Q1 | Q2 | Q3 | Total Weight |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  | 0 | 119.35 | 80.66 | 54.80 | 174.96 |
| 1 | 0 | 1.3 | 0.65 | 98.61 | 71.58 | 45.93 | 148.92 |
| 2 | 0 | 0.65 | 0.325 | 107.86 | 76.01 | 50.33 | 161.26 |
| 3 | 0.325 | 0.65 | 0.4875 | 103.15 | 73.82 | 48.18 | 155.12 |
| 4 | 0.4875 | 0.65 | 0.56875 | 100.87 | 72.71 | 47.07 | 152.04 |
| 5 | 0.56875 | 0.65 | 0.609375 | 99.74 | 72.15 | 46.51 | 150.49 |
| 6 | 0.609375 | 0.65 | 0.629688 | 99.17 | 71.87 | 46.22 | 149.71 |

At Iteration 6, the difference between the total weight and the allowable weight is 0.29 lb, less than the weight of the lightest paper (paper 1, 0.5 lb), so we stop there. Rounding down Qi’s to integers, we get Q1 = 99, Q2 = 71 and Q3 = 46. That frees up 1.25 lbs and allows the newsboy to carry an additional copy of Paper 1 and 2 each.  The final answer is Q1 = 100, Q2 = 72 and Q3 = 46.

B. Extra credit

1.

2. Show that Var[X]=E[L]⋅Var[D]+E[D]2⋅Var[L]

Proof:

Var(X)=E(X2)−[E(X)]2E[E(X2|L)]−[E(E[X|L])]2E[E(X2|L)]−E[(E[X|L])2]+Var(E[X|L])E[E(X2|L)−(E[X|L])2]+Var(E[X|L])E[Var(X|L)]+Var(E[X|L])

Var(X|L=n)=Vart=1LDt|L=nVart=1nDt|L=nVart=1nDt               (L and Dt are independent)t=1nVar[Dt]                  (Dt are independent)nVar[D]

⇒E[Var(X|L)]=E[L⋅Var(D)]=E[L]⋅Var(D)

Also,

Var(E[X|L])=Var(L⋅E[D])=E[D]2⋅Var(L)

Therefore, we get Var[X]=E[L]⋅Var[D]+E[D]2⋅Var[L].

3.  Let g(x) be the distribution function of the random demand X in the newvendor problem. Then

(c−s)E[maxQ−X,0]+(p−c)E[maxX−Q,0]=c(Q−x)g(x)dx−sE[maxQ−X,0]+(p−c)(x−Q)g(x)dxc(Q−x)g(x)dx−sE[maxQ−X,0]+p(x−Q)g(x)dxcQg(x)dx−cxg(x)dx−sE[maxQ−X,0]+pxg(x)dx−Qg(x)dxcQ−cE[X]−sE[maxQ−X,0]+pxg(x)dx+xg(x)dx−xg(x)dx−Qg(x)dxcQ−cE[X]−sE[maxQ−X,0]+pE[X]−pE[minQ,X]=-pE[minQ,X]+sE[maxQ−X,0]−cQ+(p−c)E[X]      (Equation 1)

Therefore,

min(c−s)E[maxQ−X,0]+(p−c)E[maxX−Q,0]

⇔	min −pE[minQ,X]+sE[maxQ−X,0]−cQ+(p−c)E[X]

⇔	maxpE[minQ,X]+sE[maxQ−X,0]−cQ−(p−c)E[X]

Note that the last term (p−c)E[X] can be dropped from the objective function because it is independent of Q.  Therefore, the two objectives are equivalent.

Remark: Maybe a better way to understand the result of Eq. 1, is by re-writing it as:

pE[minQ,X]+sE[maxQ−X,0]−cQ=(p−c)E[X]−(p−c)E[maxX−Q,0]−(c−s)E[maxQ−X,0]