ISyE 6201: Manufacturing Systems

Instructor : Spyros Reveliotis

Solutions for Homework #5

A.  Questions from Chapter 13

1. Manufacturing systems are generally so big and complex that the overall planning problem must be broken down somehow. Furthermore, it is clear that different types of decisions regarding the design and operation of these systems, have different time spans in terms of their implications, but they require different lead times in order to be put into effect. Finally, it is also true that different decisions require different types of data and a different level of detail regarding the modeling of the system operations, with the longer term decisions typically requiring a more aggregate perspective of the system operations compared to the shorter term ones. A hierarchical planning framework seeks to take advantage of the aforementioned effects while decomposing the overall planning problem to a number of more easily addressed sub-problems.

On the other hand, a non-hierarchical system would be one where all the different decisions under consideration would be addressed through a simple monolithic formulation. In most practical cases, such a model would be unmanageable from a computational standpoint. But even if we assume that the computational difficulties were overcome, the model might still be of limited value to the plant managers, since the underlying complexity might be overwhelming and the amount of detail in it might prevent a good understanding of the prevalent dynamics.

2. In general, the plans developed at the different levels of the hierarchical production planning framework must be revised frequently enough to take into consideration any disruptions occurring on the manufacturing shop floor (this revision introduces the necessary feedback in the planning function and keeps the plans current). Hence, the appropriate regeneration frequency will depend on the stability and the time-constants of the environment. For instance, a firm whose demand profile is stable and cycle times are long does not need to re-plan its master production schedule as frequently as does a firm with volatile demand and short cycle times.

5. Causal forecasting assumes a cause and effect relationship between the forecasted quantity and some other measurable, independent variables, while time series forecasting merely tries to extrapolate past observed trends into the future.

6. If observed data are trending upward, then an exponential smoothing model will tend to undershoot and hence exhibit negative bias (because as we explained in class, a simple exponential smoothing model essentially averages its past observations). Similarly, if the data are trending upward at an increasing rate (i.e., nonlinearly), then an exponential smoothing model with a linear trend will still lag behind and exhibit negative bias; hence, for instance, in the case of a quadratically increasing quantity, one can use a triple exponential smoothing for correcting this lag, but in general, these higher order exponential smoothing models are not used very much in practice, at least in the context of manufacturing-related applications (the book on forecasting by Makridakis is a good reference on these more advanced models).

13. Feedback is important for ensuring that the generated plans are current and feasible, especially in a hierarchical planning framework, where higher-level plans are generated based on a more aggregate / macroscopic view of the system operations. Trying to execute any given plan without feedback – generally known as an open-loop control scheme – is essentially a blind drive, and it can be successful only in (idealistic) environments with minimal stochasticity and disruptions.

B.  Problem Set:

I.   Chapter 13

1. (a) The forecasts for moving averages with m=5 and m=7 are given below. Notice that the m=7 case tends to lag the actual data more than the m=5 case. This is because the data have an upward trend (and therefore, the selection of a MA model is not a pertinent choice, in the first place!)

(b) Forecasts from exponential smoothing with  = 0.2 and  = 0.1 are given below. Notice that  = 0.1 fits worse (as judged by MSD) because this model gives more weight to past data and hence lags the upward trend more (again, Single Exponential Smoothing is not a pertinent choice for this set of data!)

(c) Using Solver in Excel, we find that  = 0.687 minimizes MSD, but  = 1 minimizes the absolute value of the Bias (it is negative because of the lag behind the data) as shown in the above table.

(d) The following gives forecasts using exponential smoothing with a linear trend with  =0.4 and  = 0.2 (note that we have used F(1)=A(1) and T(2) = A(2) –A(1) as starting points – other reasonable starting points would lead to different results particularly in the first few periods).

.

3.

There is a discontinuous jump in week 9. Evidently, they started doing something differently. If we use the outdated numbers from before week 9, it skews our forecast low for a long time.

(b)

Even with the trend, the discontinuous jump in week 9 skews our forecast low for a while. Then it causes it to overshoot for several weeks.

(c) By eyeballing the data, it seems that in general, these weekly sales evolve around a constant mean, but this mean has “jumped” in week 9 to a higher level. Hence, a simple exponential smoothing with a fairly high smoothing constant would be adequate for this situation.

II. From the table in problem 1, the sales estimates for Month 21 are 56.2 and 54.86, respectively, using the MA(5) and MA(7) methods.

As discussed in class, under the modeling assumptions underlying the MA model, the error ε(t) between demand forecast D(t)and the observed value D(t+1) is normally distributed with mean being zero and variance(1+1N)σ2, where 2 is the variance of the disturbance e in the regression model (cf. slides 10-11). Since the normal distribution is symmetric about the mean, the probability of underestimation is 50%.

Let y be the adjustment such that the probability of underestimating the sales equal 10%, i.e. PD(t+1)≤D(t)+y=1−0.1 = 0.9

Extending from the Multiple Linear Regression model as suggested in the reading part of this assignment, the quantity D(t)−D(t+1)MSE(1+1N)has a t-distribution with N-1 degrees of freedom, where N is the order of the Moving Average Model being used. We can make use of this fact to find the required adjustment y.

PD(t+1)≤D(t)+y=0.9

PD(t+1)−D(t)MSE(1+1N)≤yMSE(1+1N)=0.9

yMSE(1+1N)=t0.1,N−1

y=t0.1,N−1MSE(1+1N)

The following table summarizes the computation:

|  | D(20) | MSE | Adjustment y | Adjusted Value |
| --- | --- | --- | --- | --- |
| MA(5) | 56.20 | 15−1i=1620[Di−D(20)]2=31.7 | t 0.1, 4∙31.701+15= 9.46 | 65.66 |
| MA(7) | 54.86 | 17−1i=1420[Di−D(20)]2=30.48 | t 0.1, 6∙30.481+17= 8.50 | 63.36 |

Remark 1: Assuming that modeling assumptions underlying the MA model hold, in practice, the quantity MSE(1+1/N) is frequently approximated by MSD, where MSD is computed according to the formula in Remark 3 in slide 12.

Remark 2: I want also to reiterate that while the above discussion demonstrates the application of the theory of Confidence Intervals for the case of MA models, the particular example is rather shaky, because the underlying data has a linear trend (and as we discussed in the solution of Problem 1, MA should not have been used on this data set, in the first place.). Because of the linear trend in the data, the MSE values computed above contain also systematic variation that is unaccounted by this model, beyond the variability that is due to the disturbance e(t). This effect becomes more prominent when considering the MSD values obtained in the table provided in pg. 2 of this solution set (if there was not linear trend, these MSD values would be closer to the MSE values computed above and to each other.)

III.

- It is clear from the table in slide 19 that the seasonal averages for Years 1, 2 and 3 present a linear increase. Therefore, the seasonal average for Year 4 can be forecasted using double-exponential smoothing on these three past observations. The slope for this forecasting model is initiated to the average difference between each pair of years, i.e., T0 = [(125-100)+(153.75-125)]/2 = 26.875. The initial intercept is set I0 = 100-26.875 = 73.125

| Year | Seasonal Average | Lt(0.3) | Tt(0.2) | Ft=Lt+Tt |
| --- | --- | --- | --- | --- |
|  |  | 73.125 | 26.875 | 100.00 |
| 1 | 100.00 | 100.00 | 26.88 | 126.88 |
| 2 | 125.00 | 126.31 | 26.76 | 153.08 |
| 3 | 153.75 | 153.28 | 26.80 | 180.08 |

Multiplying the forecasted seasonal average with the seasonal indices given in slide 19, we get the seasonal forecasts for Spring, Summer, Fall and Winter equal to 157, 331, 124 and 108.

- Using Winter’s method, as demonstrated below, the seasonal demand forecasts are 149, 324, 125 and 112.

C.  Extra credit (20%)

Prove that in the case that we apply a simple exponential smoothing model on a demand series with a constant mean, as t   the mean and the variance of the forecasting error, (t), converge respectively to the values provided in item 4 of slide 14. [Hint: To get the first (resp., second) result, work with the equation provided at the top of the slide by taking the mean (resp, the variance) of both sides of this equation, and recognizing that for 0<<1,  (i) t0 as t  , and (ii) t=0t=1/(1-).]

Proof:

For t 1,   D(t) = αi=0t−1(1−α)iD(t−i)+(1−α)tD(0) 	(c.f. slide 14)

So 	E[D(t)] = Eαi=0t−1(1−α)iD(t−i)+(1−α)tD(0)

= αi=0t−1(1−α)iE[D(t−i)]+(1−α)tD(0)

=αi=0t−1(1−α)iD+(1−α)tD(0)

as t  , E[D(t)]   α11−(1−α)D+0

=D

Also, var[D(t)] = α2i=0t−1[(1−α)i]2var[D(t−i)]+(1−α)2tvar[D(0)]

=α2σ2i=0t−1[(1−α)2]i+(1−α)2tvar[D(0)]

as t  , var[D(t)]  α2σ211−(1−α)2+0

=α2σ22α−α2ασ22−α

var[D(t+1)−D(t)] = var[D(t+1)] + (-1)2 var[D(t)] =  + var[D(t)]

Hence as t  , var[D(t+1)−D(t)] σ2+ασ22−α=2σ22−α as desired.