IsyE 6202 Warehousing Systems

Fall 2002

Spyros Reveliotis

Homework #3

# Due Date: 10/10/02

(video students: 10/17/02)

- Answer questions 5.8, 5.10, 5.12, 5.13 and 5.15 in Bartholdi & Hackman’s textbook.

- Consider 3 SKU’s A, B and C whose daily requirements for storage locations follow Poisson distributions with corresponding parameters A = 10, B = 15 and C = 25. Furthermore, past studies have indicated that the daily storage space requirements of these three SKU’s are statistically independent. Since, however, items A and B tend to appear together in the received orders, the warehouse management has decided to store them together as a single class, say C1. On the other hand, item C will constitute its own class, to be called C2.

  - Determine the number of storage locations N1 and N2 to be allocated to these two classes so that the warehouse operational service level – i.e., the probability that the warehouse will not experience a storage shortage at any given day - is at least 0.95, while the employed total storage space is minimized.

  - Compare the total number of locations determined above to the minimal number of locations that would provide a service level of 95% under randomized storage.

  - Do you think that the information provided above is adequate for determining a storage layout for these two classes - i.e., the detailed assignment of the available storage locations to them - or not?

Hint: To avoid tedious computations for part (2.a), think cleverly!